Task 3 – Making measurements and adding a scale bar

- Open the image listed in the presentation. To do so, right-click the link on the presentation, go to “Copy Link Location”, then go to Import -> URL in Fiji. Do it for the image we will use: “02-Biological_Image.tif”.

- You should now have a new window in Fiji with your image.

- If you end up with an image of a clown, you have not copied the link correctly in step 1!

- Select the Line  selection tool from the main Fiji Window.

- Draw a line over your image. If you want to make perfectly horizontal/vertical lines, holding Shift makes your line change in 45-degree increments.

- Hit ‘m’ or run Analyze -> Measure. This should give you some data about the line you have drawn, such as maximum and minimum pixel values along it, and length.

- The measurements you see here can be changed by going to Analyze -> Set Measurements and picking which measures you want to calculate when using that function.

- Now, run Analyze -> Plot Profile. What do the results mean?

- Now select the Rectangle  tool. Draw a rectangle on your image and press ‘m’ (or Analyze -> Measure). What are the differences from the previous measures you have seen for your line?

- Now, we are going to make multiple selections and keep track of them.

- Make any selection you want; pick any of the selection tools you feel like using and draw a region in our image.

- Hit ‘t’ or run Edit -> Selection -> Add to Manager.

- Repeat multiple times drawing new regions every time.

- Now, on your ROI Manager window, run More -> Multi-measure. You should get measurements for every region you have drawn at once!

- For good data provenance, make sure to use More -> Save to save the regions you draw, so that later on you can see exactly which areas you have defined to achieve the measurements you obtained.

- Finally, let us add a scale bar to the image. First, we need to make sure the image is properly calibrated. Check the info bar or run Image -> Properties.

- Run Analyze -> Tools -> Scale Bar. If your calibration is correct, this should be automatically correct. Feel free to change the settings here and press ok. That is it!